#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

RF24 radio(7,8);   // CREATE THE OBJECT OF THE RF24

const byte address[][6] = {"00001","00002"};  // CREATE A PIPE I.E CHANNEL WE ARE GOING TO USE FOR COMMUNICATION. HERE WE TWO AS FOR TRANSMISSION AS WELL AS RECEPTION

void setup() {

Serial.begin(9600);   // BAUT RATE OF COMMUNICATION

radio.begin();      // START THE MODULE FOR COMMUNICATION

radio.openWritingPipe(address[1]);   // WE WILL USE ADDRESS 00002 FOR TRANSMITTING

radio.openReadingPipe(1,address[0]); // WE WILL USE ADDRESS 00001 FOR RECEIVING

radio.setPALevel(RF24_PA_MIN);     // SET THE POWER FOR DETERMINING RANGE

}

void loop() {

delay(5);

radio.stopListening();    // TO START SENDING THE DATA  WE TELL THE MODULE TO STOP TAKING THE DATA IF ANY

const char text[] = "this is Data from module 1";  // THE DATA TO BE SEND

radio.write(&text,sizeof(text));   // THIS SEND DATA TO CORRESPONDING CHANNEL OF THE OTHER NRF24 MODULE

delay(5);   // DELAY TO AVIOD ERROR

radio.startListening();   // TO LET THE MODULE TO ACCEPT THE DATA IF ANY

while(!radio.available());    // THIS CHECKS THAT IF THE MODULE IS READY TO ACCEPT OR NOT AND WAIT FOR DATA INCOMMING IN LOOP

char text1[64] = "";    // VARIABLE TO STORE THE INCOMMING DATA

radio.read(&text1, sizeof(text1));   // IT ACCEPT THE DATA FROM SENDER AND SOTRES IN VARIABLE TEXT1

Serial.println(text1);   // PRINT THE DATA THAT IS RECEIVED

delay(1000);   // FOR PROPER PRESENTATION OF DATA

}