Code for transeceiver for 2

#include <SPI.h>

#include <nRF24L01.h>

#include <RF24.h>

RF24 radio(7,8);  // CREATE THE OBJECT OF THE RF24

const byte address[][6] = {"00001","00002"};  //  CREATE A PIPE I.E CHANNEL WE ARE GOING TO USE FOR COMMUNICATION. HERE WE TWO AS FOR TRANSMISSION AS WELL AS RECEPTION

void setup() {

Serial.begin(9600);  // BAUT RATE OF COMMUNICATION

radio.begin();    // START THE MODULE FOR COMMUNICATION

radio.openWritingPipe(address[0]);  // WE WILL USE ADDRESS 00002 FOR TRANSMITTING

radio.openReadingPipe(1,address[1]); // WE WILL USE ADDRESS 00001 FOR RECEIVING

radio.setPALevel(RF24_PA_MIN);   // WE WILL USE ADDRESS 00001 FOR RECEIVING

}

void loop() {

delay(5);

radio.startListening(); // TO LET THE MODULE TO ACCEPT THE DATA IF ANY

if(radio.available()){        // THIS CHECKS THAT IF THE MODULE IS READY TO ACCEPT OR NOT

char text3[64] = "";     // VARIABLE TO STORE THE INCOMMING DATA

radio.read(&text3, sizeof(text3));    // IT ACCEPT THE DATA FROM SENDER AND SOTRES IN VARIABLE TEXT1

Serial.println(text3); // PRINT THE DATA THAT IS RECEIVED

delay(1000);   // FOR PROPER PRESENTATION OF DATA

}

delay(5);

radio.stopListening(); // TO START SENDING THE DATA  WE TELL THE MODULE TO STOP TAKING THE DATA IF ANY

const char text2[] = "this is data from module 2"; // THE DATA TO BE SEND

radio.write(&text2,sizeof(text2)); // THIS SEND DATA TO CORRESPONDING CHANNEL OF THE OTHER NRF24 MODULE

delay(100);   // DELAY TO AVIOD ERROR

}