Code for Secure Passway using the fingerprint Scanner

/* AIM:- INTERFACING THE FINGERPRINT SCANNER R307 AND TEST FINGER RECOGNITION BY GETTING A VALUE IF THE FINGER IS RECOGNISED PROPERLY

* FINGERPRINT SCANNER HAVE BECOME VERY FAMILIER NOW A DAY SO IT IS IMFORTANT TO LEARN ITS WORKING

* FINGERPRINT SCANNER GENERATE THE IMAGE OF THE RIDGES AND VALLEYS THAT MAKE UP THE FINGERPRINT

* FINGERPRINT SCANNER R307 SENDS A OPTICAL LIGHT TO COLLECT THE IMAGE OF THE RIBS AND VALLEYS OF THE FINGERPRINT

* IN THIS PROGRAM THE FINGER MATCHING IS DONE AND A CONFIDENCE VALUE OR A SURETY VALUE IS PROVIDED IT LET US KNOW THE MATCHING OF THE FINGERPRINT IN NUMBERS

* COMPONENTS USED:- UNIKO EKAM BOARD(ARDUINO) ,FINGERPRINT SCANNER R307 ,CONNECTER FOR SCANNER ,JUMPER WIRE

* CONNECTION:- PIN NUMBERED 1 OF THE FINGERPRINT SCANNER IS CONNECTED TO THE +5 VOLTS OF BOARD

*              PIN NUMBERED 2 OF THE FINGERPRINT SCANNER IS CONNECTED TO THE GND(0 VOLTS) OF BOARD

*              PIN NUMBERED 3 OF THE FINGERPRINT SCANNER IS CONNECTED TO THE D2 OF BOARD

*              PIN NUMBERED 4 OF THE FINGERPRINT SCANNER IS CONNECTED TO THE D3 OF BOARD

* R307 HAS TWO MORE PINS WHICH IS NOT USED FREQUENTLY THE PIN IS NAMED AS 5.TOUCH 6.VT

* FOR THIS PROGRAM WE WILL USE ONLY THE FIRST 4 PINS

* PRECAUTION:- FINGERPRINT WILL NOT BE RECONGNISED IF THE FINGER WAS UNDER WATER FOR SOME TIME

* REFERENCES:- LEARN.ADAFRUIT.COM ,DATASHEET ,COMPUTER.HOWSHUFFWORKS.COM/FINGERPRINT-SCANNER3

* DEVELOPED BY :- PARTH JOSHI AND Limor Fried/Ladyada(Adafruit)

* LIBRABRY  USED - Adafruit_Fingerprint.h ,SoftwareSerial.h

* date

*/

#include <SoftwareSerial.h>

#include <Adafruit_Fingerprint.h>

// For UNO and others without hardware serial, we use software serial //

SoftwareSerial mySerial(2, 3); // IT VIRTUALLY CREATED THE SERIAL COMMUNICATION TX AND RX AVAILABLE ON THE DIGITAL PIN D2(TX) AND D3(RX)

Adafruit_Fingerprint finger = Adafruit_Fingerprint(&mySerial); //CREATE THE OBJECT OF THE ADAFRUIT_FINGERPRINT AS FINGER TO ACESS THE METHOD OF THE LIBRABRY

int ir1;

int c;

void setup()

{

Serial.begin(9600);  // SETTING THE BAUT RATE OF THE COMMUNICATION

pinMode(8,OUTPUT);

pinMode(9,OUTPUT);

pinMode(A0,INPUT);

pinMode(7,OUTPUT);

while (!Serial);     // CHECKS WHETHER A SPECFIED PORT IS AVAILABLE FOR THE COMMUNICATION OR NOT

delay(100);

Serial.println("\n\n finger detect test");

// set the data rate for the sensor serial port

finger.begin(57600);

if (finger.verifyPassword()) {        // THERE IS THE HANDSHAKE BETWEEN THE FINGERPRINT SCANNER AND THE ARDUINO AND IF THE HASDSHAKE IS SUCCESSFUL THEN IT PRINTS SENSOR FOUND

Serial.println("Found fingerprint sensor!");

} else {

Serial.println("Did not find fingerprint sensor :(");

while (1) { delay(1); }  // THE MESSAGE DID NOT FOUND CONTINUES TO STAY UNTIL THE VALID HANDSHAKE IS PERFORMED

}

finger.getTemplateCount();   // THIS FUNCTION GIVES THE NUMBER OF TEMPLETES STORED IN THE DATABASE OF THE SENSOR

Serial.print("Sensor contains "); Serial.print(finger.templateCount); Serial.println(" templates");

Serial.println("Waiting for valid finger...");  // AFTER GIVING THE COUNT OF THE TEMPLETES STORED IT TELL THE USER TO ENTER THE VALID FINGER STORED IN THE SENSOR MEMORY

}

void loop()                     // run over and over again

{

getFingerprintIDez();

delay(50);            //don't ned to run this at full speed.

ir1 = analogRead(A0);

if(ir1 < 100 )

{

c++;

delay(500);

}

if(c > 10)

{

digitalWrite(7,HIGH);

delay(4000);

digitalWrite(7,LOW);

c = 0;

}

}

uint8_t getFingerprintID() {    // THIS FUNCTION COLLECTS THE FINGERPRINT SO THE IT CAN CHECK WHETHER THE VALID FINGERPRINT IS PRESENT OR NOT

uint8_t p = finger.getImage();  // GET THE IMAGE OF THE FINGERPRINT

switch (p) {   // THE IMAGE COLLECTED IS CHECKED FOR ANY ERROR PRESENT

case FINGERPRINT_OK:

Serial.println("Image taken");

break;

case FINGERPRINT_NOFINGER:

Serial.println("No finger detected");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

return p;

case FINGERPRINT_IMAGEFAIL:

Serial.println("Imaging error");

return p;

default:

Serial.println("Unknown error");

return p;

}

// OK success!

p = finger.image2Tz();    // THE COLLECTED IMAGE IF NO ERROR PRESENT IS CONVERTED INTO THE CHARECTER FILE

switch (p) {    // THE CONVERTED FILE IS  CHECKED FOR ANY ERROR PRESENT OR NOT

case FINGERPRINT_OK:

Serial.println("Image converted");

break;

case FINGERPRINT_IMAGEMESS:

Serial.println("Image too messy");

return p;

case FINGERPRINT_PACKETRECIEVEERR:

Serial.println("Communication error");

return p;

case FINGERPRINT_FEATUREFAIL:

Serial.println("Could not find fingerprint features");

return p;

case FINGERPRINT_INVALIDIMAGE:

Serial.println("Could not find fingerprint features");

return p;

default:

Serial.println("Unknown error");

return p;

}

// OK converted!

p = finger.fingerFastSearch();   // NOW THE FINGERPRINT IS SEARCHED IN THE MEMORY OF THE SENSOR  IF IT IS MATCHED THEN THE MESSEAGE IS PRINTED ON THE SERIAL MONITER TELLS ITS A SUCCESS

if (p == FINGERPRINT_OK) {  // THE SEARCHED FINGERPRINT AND THE COLLECTED FINGERPRINT IS CHECKED AND SUITABLE MESSAGE IS DISPLAYED

Serial.println("Found a print match!");

} else if (p == FINGERPRINT_PACKETRECIEVEERR) {

Serial.println("Communication error");

return p;

} else if (p == FINGERPRINT_NOTFOUND) {

Serial.println("Did not find a match");

return p;

} else {

Serial.println("Unknown error");

return p;

}

// found a match!

Serial.print("Found ID #"); Serial.print(finger.fingerID);

Serial.print(" with confidence of "); Serial.println(finger.confidence);

return finger.fingerID;   // IT IS USED TO SIGNIFY THE FINGERPRINT OF THE PARTICULAR ID IS CHECKED

}

// returns -1 if failed, otherwise returns ID #

int getFingerprintIDez() {  // THIS FUNCTION IS CREATED TO TERMINATE THE FUNCTION ABOVE CREATED IF ANY ERROR ENCOUNTERED IN ITS FUNCTIONALITY

uint8_t p = finger.getImage();    // THE FINGERPRINT IMAGE IS STORED IN THE VARIABLE P AND IT IS CHECKED AGAINTS THE ABOVE FUNCTIONS VALUES IF IT DOESNT MATCHES THEN P IS RETURNED AS -1

if (p != FINGERPRINT_OK)  return -1;

p = finger.image2Tz();

if (p != FINGERPRINT_OK)  return -1;

p = finger.fingerFastSearch();

if (p != FINGERPRINT_OK)  return -1;

// found a match!

Serial.print("Found ID #"); Serial.print(finger.fingerID);

Serial.print(" with confidence of "); Serial.println(finger.confidence); // IT IS FUNCTION OF ADAFRUIT FINGERPRINT LIBRARY WHICH GIVES THE VALUE OF CONFIDENCE OF THE MATCHED FINGERPRINT

if(finger.confidence > 100){

digitalWrite(8, HIGH);

digitalWrite(9, LOW);

delay(13000);

digitalWrite(8, LOW);

digitalWrite(9, LOW);

delay(5000);

digitalWrite(8, LOW);

digitalWrite(9, HIGH);

delay(13000);

digitalWrite(8, LOW);

digitalWrite(9, LOW);

}

return finger.fingerID; // IT IS USED TO SIGNIFY THE FINGERPRINT OF THE PARTICULAR ID IS CHECKED

}